# ENVISION ONE SOP

V2.0

Things you’ll need:

- PPE

- Resin filter

- 16oz cup

- Resin

Layout

Training is required prior to operating Envision One printers.  This guide is only for operators who have been trained.   Casts require E cast resin setup and E cast RFID card.  Models require E model resin and E model RFID card.  Cleaning, setup, and priming process must be learned via practical training.  It will be assumed you have been trained if you are using this documentation.

Prior to first use, be sure to check resin.  If necessary, filter and top prior to use.  If you remove the resin tray, be sure to prime the O2 again before printing (**priming sequence is part of training**).  A full resin tray should look like this:

DO NOT OVERFILL.  Resin must roll on mixing station for an hour.

CAST PRINT DIRECTIONS

Nesting

Create new project in envisionone app and be sure material is set to E cast.

F7 all models prior to nesting.  Center, flip 180, and add supports TO ONE MODEL AT A TIME PER TRAINING.

Be sure beam spacing is set to 3.9 prior to adding supports for casts .

Verify proper print by viewing slices prior to printing.  This is part of training.

Send job to printer after you change the name to our naming convention.  This is part of training.

Save job after sent to printer in the envision folder with proper naming convention.

What not to do

Do NOT try to nest more than one file at a time.

Do NOT forget to check resin for debris and level prior to printing.

Best practices

Nest one frame at a time.

Add supports to areas with known issues (origin points, pads, clasps).

Get someone to double check your work.

Don’t rush.

Spend time up front so you don’t have to do costly reprints later.

MODEL PRINT DIRECTIONS

F7 all models prior to nesting.

Be sure beam spacing is set to 1.0 prior to adding supports for casts .

Verify proper print by viewing slices prior to printing.  This is part of training.

Send job to printer after you change the name to our naming convention.  This is part of training.

Save job after sent to printer in the envision folder with proper naming convention.

What not to do

Do NOT try to nest more than one file at a time.

Do NOT forget to check resin for debris and level prior to printing.

Best practices

Nest one model at a time.

Verify correct beam spacing

Get someone to double check your work.

Don’t rush.

Spend time up front so you don’t have to do costly reprints later.

POST PROCESSING DIRECTIONS

Casts and models must be removed from the build plate.  Proper training is required prior to attempting removal of prints from Envisiontec build plates.

Upon removal, the casts/model should be placed in 99% alcohol baths.  The casts will have one set of baths and the models will have a different set.  They are NOT interchangeable.  Please be sure cast baths are labeled “Dirty Cast” and “Cast Clean”.

Are Casts require 60 seconds per bath of gentle agitation by hand per training in each bath.

Models require 180 seconds in the ultrasonic per each bath per training.

Casts require a cure session at 90 seconds on the actual model with all supports intact.

Models require two 180 second sessions with supports intact.

What not to do

Don’t put casts in the ultra-sonic.

Don’t forget the casts / models in the alcohol baths.

Don’t over cure

Best practices

Pay attention to the details.

Do the process until it comes out clean.

Follow the directions and your training.

If you are unsure, stop and ask questions.